| ECN NUMBER: TS030123_Acbebis DATE: January. 23. 2003 |  |  |
| --- | --- | --- |
| MODEL: SN-324B (Combo Slim Drive) |  | INITIATED BY: YC Chang |
| PRODUCT FAMILY: COMBO Slim Drive |  |  |
| CHANGE CODE : 1 - FUNCTIONAL FAILURE 2 - DESIGN ENHANCEMENT 3 - COST REDUCTION 4 - COMPONENT SHORTAGE 5 – OTHER |  |  |
| REASON(S) FOR CHANGE(S): CODE – 2 Firmware upgrade (U101 -> U102 , Capital Letter) |  |  |
| DECRIPTION(S) FOR CHANGE(S): The firmware has been upgraded in order to improve the drive’s performance as below. - Modify the Get Configure and GESN for HCT 11.0 MMC command test item. - Added the Write Strategy : CD-R (7 kinds) -> TDK, MPO (75min) / GIGA, PLASMON, FORNET, BEALL, RITEK (80min) : CD-RW (3 kinds) -> GIGA, Digital ST, NANYA 3) Improved an audio scratch (2.2mm) when DAE (Digital Audio Extraction). : We changed DAE (Digital Audio Extraction) speed : CAV 16x -> Inner area CLV 8x speed, after 0x8000 LBA, CAV 16x speed. - Modify the DVD Seek retry algorithm when focus-drop is occurred for DVD Disc. - Modify the DVD Read Speed to CLV 2x so as improve more accurately the readability for high unbalanced DVD Disc. - Modify the audio pop-noise when audio playing back in specific condition. : It caused the read track information command during audio play back or DAE (Digital Audio Extraction) in Specific PC System installed Roxio. |  |  |
| Before Change(s): Model: SN-324B/ACT F/W Rev: U101 (Capital Letter) H/W Rev: A Other: N/A | After Change(s): Model: SN-324B/ACT F/W Rev: U102 (Capital Letter) H/W Rev: A Other: N/A |  |